一、设计一个输入输出的综合项目实例，使用BTN_LEFT按键翻转红色LED_R灯、 BTN_RIGHT按键翻转蓝色LED_B灯，按下BTN_UP按键发出5s救护车报警声，按下BTN_DOWN键按发出5s电动车报警声。

要求：采用中断的方式检测按键状态，且按键带有消抖功能；

void MX_GPIO_Init(void)

{

GPIO_InitTypeDef GPIO_InitStruct = {0};

/* GPIO Ports Clock Enable */

__HAL_RCC_GPIOC_CLK_ENABLE();

__HAL_RCC_GPIOH_CLK_ENABLE();

__HAL_RCC_GPIOB_CLK_ENABLE();

__HAL_RCC_GPIOF_CLK_ENABLE();

/*Configure GPIO pin Output Level */

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_0, GPIO_PIN_RESET);

/*Configure GPIO pin Output Level */

HAL_GPIO_WritePin(GPIOF, GPIO_PIN_11|GPIO_PIN_12, GPIO_PIN_RESET);

/*Configure GPIO pins : PC0 PC4 */

GPIO_InitStruct.Pin = GPIO_PIN_0|GPIO_PIN_4;

GPIO_InitStruct.Mode = GPIO_MODE_IT_FALLING;

GPIO_InitStruct.Pull = GPIO_PULLUP;

HAL_GPIO_Init(GPIOC, &GPIO_InitStruct);

/*Configure GPIO pins : PC1 PC5 */

GPIO_InitStruct.Pin = GPIO_PIN_1|GPIO_PIN_5;

GPIO_InitStruct.Mode = GPIO_MODE_IT_FALLING;

GPIO_InitStruct.Pull = GPIO_PULLUP;

HAL_GPIO_Init(GPIOC, &GPIO_InitStruct);

/*Configure GPIO pin : PB0 */

GPIO_InitStruct.Pin = GPIO_PIN_0;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

HAL_GPIO_Init(GPIOB, &GPIO_InitStruct);

/*Configure GPIO pins : PF11 PF12 */

GPIO_InitStruct.Pin = GPIO_PIN_11|GPIO_PIN_12;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

HAL_GPIO_Init(GPIOF, &GPIO_InitStruct);

HAL_NVIC_SetPriority(EXTI0_IRQn, 7, 0);

HAL_NVIC_EnableIRQ(EXTI0_IRQn);

HAL_NVIC_SetPriority(EXTI4_IRQn, 7, 0);

HAL_NVIC_EnableIRQ(EXTI4_IRQn);

HAL_NVIC_SetPriority(EXTI1_IRQn, 7, 0);

HAL_NVIC_EnableIRQ(EXTI1_IRQn);

HAL_NVIC_SetPriority(EXTI9_5_IRQn, 7, 0);

HAL_NVIC_EnableIRQ(EXTI9_5_IRQn);

}

volatile uint8_t button_up = 0;

volatile uint8_t button_down = 0;

void delay(uint32_t i)

{

while(i--);

}

void sound1(void)

{

uint32_t i=30000;

while(i)

{

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_0, GPIO_PIN_SET);

delay(i);

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_0, GPIO_PIN_RESET);

delay(i);

i = i - 6;

}

}

void sound2(void)

{

uint32_t i=6000;

while(i)

{

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_0, GPIO_PIN_SET);

delay(i);

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_0, GPIO_PIN_RESET);

delay(i);

i = i - 6;

}

}

//外部中断回调函数

void HAL_GPIO_EXTI_Callback(uint16_t GPIO_Pin)

{

HAL_Delay(15);

if(HAL_GPIO_ReadPin(GPIOC, GPIO_Pin) == GPIO_PIN_RESET)

{

if(GPIO_Pin == GPIO_PIN_0)

{

HAL_GPIO_TogglePin(GPIOF, GPIO_PIN_12);	// 翻转红色灯

}

if(GPIO_Pin == GPIO_PIN_4)

{

HAL_GPIO_TogglePin(GPIOF, GPIO_PIN_11);	// 翻转绿色灯

}

if(GPIO_Pin == GPIO_PIN_5)

{

button_up=1;

}

if(GPIO_Pin == GPIO_PIN_1)

{

button_down=1;

}

}

}

int main(void)

{

HAL_Init();

SystemClock_Config();

MX_GPIO_Init();

while (1)

{

if (button_up)

{

button_up = 0;

for(int i=0;i<50;i++) sound2();

}

if (button_down)

{

button_down = 0;

for(int i=0;i<3;i++) sound1();

}

}

}

二、设计一个输入输出的综合项目实例，使用BTN_LEFT按键翻转红色LED_R灯、 BTN_RIGHT按键翻转绿色LED_B灯，按下BTN_UP按键发出5s救护车报警声，按下BTN_DOWN键按发出5s电动车报警声。

要求：按键带有消抖功能；

void MX_GPIO_Init(void)

{

GPIO_InitTypeDef GPIO_InitStruct = {0};

__HAL_RCC_GPIOC_CLK_ENABLE();

__HAL_RCC_GPIOH_CLK_ENABLE();

__HAL_RCC_GPIOB_CLK_ENABLE();

__HAL_RCC_GPIOF_CLK_ENABLE();

__HAL_RCC_GPIOA_CLK_ENABLE();

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_0, GPIO_PIN_RESET);

HAL_GPIO_WritePin(GPIOF, GPIO_PIN_11|GPIO_PIN_12, GPIO_PIN_SET);

GPIO_InitStruct.Pin = GPIO_PIN_0|GPIO_PIN_1|GPIO_PIN_4|GPIO_PIN_5;

GPIO_InitStruct.Mode = GPIO_MODE_INPUT;

GPIO_InitStruct.Pull = GPIO_PULLUP;

HAL_GPIO_Init(GPIOC, &GPIO_InitStruct);

/*Configure GPIO pin : PB0 */

GPIO_InitStruct.Pin = GPIO_PIN_0;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_PULLUP;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_HIGH;

HAL_GPIO_Init(GPIOB, &GPIO_InitStruct);

/*Configure GPIO pins : PF11 PF12 */

GPIO_InitStruct.Pin = GPIO_PIN_11|GPIO_PIN_12;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

HAL_GPIO_Init(GPIOF, &GPIO_InitStruct);

}

void delay(uint32_t i)

{

while(i--);

}

void sound1(void)

{

uint32_t i=30000;

while(i)

{

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_0, GPIO_PIN_SET);

delay(i);

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_0, GPIO_PIN_RESET);

delay(i);

i = i - 6;

}

}

void sound2(void)

{

uint32_t i=6000;

while(i)

{

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_0, GPIO_PIN_SET);

delay(i);

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_0, GPIO_PIN_RESET);

delay(i);

i = i - 6;

}

}

int main(void)

{

HAL_Init();

SystemClock_Config();

MX_GPIO_Init();

while (1)

{

/* USER CODE END WHILE */

// 读取 GPIOC 的输入数据寄存器 IDR

uint32_t pinState = GPIOC->IDR;

// 检查每个按钮的状态

if ((pinState & GPIO_PIN_0) == GPIO_PIN_RESET)  // 按下了 GPIO_BTN_LEFT 按键

{

HAL_Delay(200);

HAL_GPIO_TogglePin(GPIOF, GPIO_PIN_12);  // 翻转红色LED_R灯

}

else if ((pinState & GPIO_PIN_1) == GPIO_PIN_RESET)  // 按下了 GPIO_BTN_DOWN 按键

{

HAL_Delay(200);

for (int i = 50; i > 0; i--)

{

sound2();  // 5s电动车报警声

}

}

else if ((pinState & GPIO_PIN_4) == GPIO_PIN_RESET)  // 按下了 GPIO_BTN_RIGHT 按键

{

HAL_Delay(200);

HAL_GPIO_TogglePin(GPIOF, GPIO_PIN_11);  // 翻转蓝色LED_B灯

}

else if ((pinState & GPIO_PIN_5) == GPIO_PIN_RESET)  // 按下了 GPIO_BTN_UP 按键

{

HAL_Delay(200);

sound1();

sound1();  // 5s救护车声音

}

}

}

三、用串口1来进行异步串行通信，发送数据时采用状态查询方式进行发送，接收数据时（长度不超过200字节）采用中断方式进行接收，当收到回车、换行符时，将收到的字符串通过串口返回给PC。

void MX_GPIO_Init(void)

{

/* GPIO Ports Clock Enable */

__HAL_RCC_GPIOC_CLK_ENABLE();

__HAL_RCC_GPIOH_CLK_ENABLE();

__HAL_RCC_GPIOA_CLK_ENABLE();

}

void MX_USART1_UART_Init(void)

{

huart1.Instance = USART1;

huart1.Init.BaudRate = 115200;

huart1.Init.WordLength = UART_WORDLENGTH_8B;

huart1.Init.StopBits = UART_STOPBITS_1;

huart1.Init.Parity = UART_PARITY_NONE;

huart1.Init.Mode = UART_MODE_TX_RX;

huart1.Init.HwFlowCtl = UART_HWCONTROL_NONE;

huart1.Init.OverSampling = UART_OVERSAMPLING_16;

if (HAL_UART_Init(&huart1) != HAL_OK)

{

Error_Handler();

}

}

#define USART_REC_LEN 200  // 定义接收缓冲区长度

#define RXBUFFERSIZE 1     // 每次接收一个字节

uint8_t  g_usart_rx_buf[USART_REC_LEN];  /* 接收缓冲,最大USART_REC_LEN个字节. */

uint8_t  g_rx_buffer[RXBUFFERSIZE];      /* HAL库USART接收Buffer */

uint8_t  g_usart_rx_cnt = 0;             /* HAL库USART接收字节CNT */

uint8_t     flag = 0;     /* 接收完成标志 */

void HAL_UART_RxCpltCallback(UART_HandleTypeDef *huart)

{

if(huart->Instance == USART1)

{

if (g_rx_buffer[0] == '\r' || g_rx_buffer[0] == '\n')

{

flag = 1;

}

else

{

g_usart_rx_buf[g_usart_rx_cnt] = g_rx_buffer[0];

g_usart_rx_cnt++;

if(g_usart_rx_cnt >= USART_REC_LEN)

{

g_usart_rx_cnt = 0;  // 防止缓冲区溢出

}

}

}

}

int main(void)

{

HAL_Init();

SystemClock_Config();

MX_GPIO_Init();

MX_USART1_UART_Init();

HAL_UART_Receive_IT(&huart1, g_rx_buffer, RXBUFFERSIZE);

while (1)

{

if (flag)

{

// 发送接收到的数据

HAL_UART_Transmit(&huart1, g_usart_rx_buf, g_usart_rx_cnt, 1000);

g_usart_rx_cnt = 0;  // 重置接收计数器

flag = 0;

}

}

}

四、通过I2C2总线读写AP3216C传感器采集的环境光数据和距离数据，并将它们显示在LCD上，数据保留一位小数。要求：采用I2C2的HAL库驱动函数(不是GPIO口模拟I2C总线)进行操作AP3216C；

void MX_GPIO_Init(void)

{

GPIO_InitTypeDef GPIO_InitStruct = {0};

__HAL_RCC_GPIOC_CLK_ENABLE();

__HAL_RCC_GPIOF_CLK_ENABLE();

__HAL_RCC_GPIOH_CLK_ENABLE();

__HAL_RCC_GPIOE_CLK_ENABLE();

__HAL_RCC_GPIOD_CLK_ENABLE();

__HAL_RCC_GPIOA_CLK_ENABLE();

__HAL_RCC_GPIOG_CLK_ENABLE();

HAL_GPIO_WritePin(GPIOD, GPIO_PIN_3, GPIO_PIN_SET);

GPIO_InitStruct.Pin = KEY_LEFT_Pin|KEY_DOWN_Pin|KEY_RIGHT_Pin|KEY_UP_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_INPUT;

GPIO_InitStruct.Pull = GPIO_PULLUP;

HAL_GPIO_Init(GPIOC, &GPIO_InitStruct);

GPIO_InitStruct.Pin = GPIO_PIN_3;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_PULLUP;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_MEDIUM;

HAL_GPIO_Init(GPIOD, &GPIO_InitStruct);

}

void MX_I2C2_Init(void)

{

hi2c2.Instance = I2C2;

hi2c2.Init.ClockSpeed = 100000;

hi2c2.Init.DutyCycle = I2C_DUTYCYCLE_2;

hi2c2.Init.OwnAddress1 = 0;

hi2c2.Init.AddressingMode = I2C_ADDRESSINGMODE_7BIT;

hi2c2.Init.DualAddressMode = I2C_DUALADDRESS_DISABLE;

hi2c2.Init.OwnAddress2 = 0;

hi2c2.Init.GeneralCallMode = I2C_GENERALCALL_DISABLE;

hi2c2.Init.NoStretchMode = I2C_NOSTRETCH_DISABLE;

if (HAL_I2C_Init(&hi2c2) != HAL_OK)

{

Error_Handler();

}

}

void MX_TIM14_Init(void)

{

TIM_OC_InitTypeDef sConfigOC = {0};

htim14.Instance = TIM14;

htim14.Init.Prescaler = 21-1;

htim14.Init.CounterMode = TIM_COUNTERMODE_UP;

htim14.Init.Period = 4000-1;

htim14.Init.ClockDivision = TIM_CLOCKDIVISION_DIV1;

htim14.Init.AutoReloadPreload = TIM_AUTORELOAD_PRELOAD_DISABLE;

if (HAL_TIM_Base_Init(&htim14) != HAL_OK)

{

Error_Handler();

}

if (HAL_TIM_PWM_Init(&htim14) != HAL_OK)

{

Error_Handler();

}

sConfigOC.OCMode = TIM_OCMODE_PWM1;

sConfigOC.Pulse = 3200;

sConfigOC.OCPolarity = TIM_OCPOLARITY_HIGH;

sConfigOC.OCFastMode = TIM_OCFAST_DISABLE;

if (HAL_TIM_PWM_ConfigChannel(&htim14, &sConfigOC, TIM_CHANNEL_1) != HAL_OK)

{

Error_Handler();

}

HAL_TIM_MspPostInit(&htim14);

}

void MX_USART1_UART_Init(void)

{

huart1.Instance = USART1;

huart1.Init.BaudRate = 115200;

huart1.Init.WordLength = UART_WORDLENGTH_8B;

huart1.Init.StopBits = UART_STOPBITS_1;

huart1.Init.Parity = UART_PARITY_NONE;

huart1.Init.Mode = UART_MODE_TX_RX;

huart1.Init.HwFlowCtl = UART_HWCONTROL_NONE;

huart1.Init.OverSampling = UART_OVERSAMPLING_16;

if (HAL_UART_Init(&huart1) != HAL_OK)

{

Error_Handler();

}

}

uint8_t AP3216C_Init(void)

{

uint8_t temp = 0;

//初始化IIC

AP3216C_WriteOneByte(0x00, 0X04);	//复位AP3216C

HAL_Delay(50);						//AP33216C复位至少10ms

AP3216C_WriteOneByte(0x00, 0X03);	//开启ALS、PS+IR

temp = AP3216C_ReadOneByte(0X00);		//读取刚刚写进去的0X03

if(temp == 0X03)return 0;				//AP3216C正常

else return 1;						//AP3216C失败

}

void AP3216C_ReadData(uint16_t* ir, uint16_t* ps, uint16_t* als)

{

uint8_t buf[6];

uint8_t i;

for(i = 0; i < 6; i++)

{

buf[i] = AP3216C_ReadOneByte(0X0A + i);		//循环读取所有传感器数据

}

if(buf[0] & 0X80)*ir = 0;						//IR_OF位为1,则数据无效

else *ir = ((uint16_t)buf[1] << 2) | (buf[0] & 0X03); 	//读取IR传感器的数据

*als = ((uint16_t)buf[3] << 8) | buf[2];				//读取ALS传感器的数据

if(buf[4] & 0x40)*ps = 0;    					//IR_OF位为1,则数据无效

else *ps = ((uint16_t)(buf[5] & 0X3F) << 4) | (buf[4] & 0X0F); //读取PS传感器的数据

}

uint8_t AP3216C_WriteOneByte(uint8_t reg, uint8_t data)

{

return HAL_I2C_Mem_Write(&hi2c2, AP3216C_ADDR, reg, I2C_MEMADD_SIZE_8BIT, &data, 1, 100);

}

uint8_t AP3216C_ReadOneByte(uint8_t reg)

{

uint8_t read_data;

HAL_I2C_Mem_Read(&hi2c2, AP3216C_ADDR, reg, I2C_MEMADD_SIZE_8BIT, &read_data, 1, 100);

return read_data;

}

int fputc(int ch, FILE *f)

{

HAL_UART_Transmit(&huart1, (uint8_t *)&ch, 1, 1000);

return ch;

}

float ALSValue;

float PS_RAW;

float IR_RAW;

char display_str[50];//显示字符串

int main(void)

{

uint16_t ir, als, ps;

HAL_Init();

SystemClock_Config();

MX_GPIO_Init();

MX_FSMC_Init();

MX_TIM14_Init();

MX_USART1_UART_Init();

MX_I2C2_Init();

HAL_TIM_Base_Start_IT(&htim14);

HAL_TIM_PWM_Start(&htim14, TIM_CHANNEL_1);

drv_lcd_init();

lcd_set_color(WHITE, BLACK);

while (AP3216C_Init()) /* 检测不到AP3216C */

{

sprintf(display_str, "AP3216C Check Failed!   ");

lcd_show_string(20, 180, 24, display_str);

HAL_Delay(500);

}

sprintf(display_str, "AP3216C Check OK   ");

lcd_show_string(20, 180, 24, display_str);

while (1)

{

AP3216C_ReadData(&ir, &ps, &als);		//读取数据

ALSValue = als * 0.36;// Lux = 16 bit ALS data * Resolution

PS_RAW = ps;

IR_RAW = ir;

sprintf(display_str, "IR:%.1f    ", IR_RAW);

lcd_show_string(20, 40, 24, display_str);

sprintf(display_str, "PS:%.1f    ", PS_RAW);

lcd_show_string(20, 85, 24, display_str);

sprintf(display_str, "ALS:%.1f    ", ALSValue);

lcd_show_string(20, 130, 24, display_str);

HAL_Delay(1000);

}

五、采集内部温度传感器、内部参考电压、Vbat共3个通道模拟电压信息，并将电压值（保留3为小数）显示在LCD上；同时将内部温度传感器电压转换为温度显示在LCD上。

void MX_GPIO_Init(void)

{

GPIO_InitTypeDef GPIO_InitStruct = {0};

/* GPIO Ports Clock Enable */

__HAL_RCC_GPIOC_CLK_ENABLE();

__HAL_RCC_GPIOF_CLK_ENABLE();

__HAL_RCC_GPIOH_CLK_ENABLE();

__HAL_RCC_GPIOE_CLK_ENABLE();

__HAL_RCC_GPIOD_CLK_ENABLE();

__HAL_RCC_GPIOA_CLK_ENABLE();

__HAL_RCC_GPIOG_CLK_ENABLE();

/*Configure GPIO pin Output Level */

HAL_GPIO_WritePin(GPIOD, GPIO_PIN_3, GPIO_PIN_SET);

/*Configure GPIO pin : PD3 */

GPIO_InitStruct.Pin = GPIO_PIN_3;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_PULLUP;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_MEDIUM;

HAL_GPIO_Init(GPIOD, &GPIO_InitStruct);

}

void MX_ADC1_Init(void)

{

ADC_ChannelConfTypeDef sConfig = {0};

hadc1.Instance = ADC1;

hadc1.Init.ClockPrescaler = ADC_CLOCK_SYNC_PCLK_DIV6;

hadc1.Init.Resolution = ADC_RESOLUTION_12B;

hadc1.Init.ScanConvMode = ENABLE;

hadc1.Init.ContinuousConvMode = DISABLE;

hadc1.Init.DiscontinuousConvMode = DISABLE;

hadc1.Init.ExternalTrigConvEdge = ADC_EXTERNALTRIGCONVEDGE_NONE;

hadc1.Init.ExternalTrigConv = ADC_SOFTWARE_START;

hadc1.Init.DataAlign = ADC_DATAALIGN_RIGHT;

hadc1.Init.NbrOfConversion = 3;

hadc1.Init.DMAContinuousRequests = ENABLE;

hadc1.Init.EOCSelection = ADC_EOC_SINGLE_CONV;

if (HAL_ADC_Init(&hadc1) != HAL_OK)

{

Error_Handler();

}

/** Configure for the selected ADC regular channel its corresponding rank in the sequencer and its sample time.

*/

sConfig.Channel = ADC_CHANNEL_TEMPSENSOR;

sConfig.Rank = 1;

sConfig.SamplingTime = ADC_SAMPLETIME_56CYCLES;

if (HAL_ADC_ConfigChannel(&hadc1, &sConfig) != HAL_OK)

{

Error_Handler();

}

/** Configure for the selected ADC regular channel its corresponding rank in the sequencer and its sample time.

*/

sConfig.Channel = ADC_CHANNEL_VREFINT;

sConfig.Rank = 2;

if (HAL_ADC_ConfigChannel(&hadc1, &sConfig) != HAL_OK)

{

Error_Handler();

}

/** Configure for the selected ADC regular channel its corresponding rank in the sequencer and its sample time.

*/

sConfig.Channel = ADC_CHANNEL_VBAT;

sConfig.Rank = 3;

if (HAL_ADC_ConfigChannel(&hadc1, &sConfig) != HAL_OK)

{

Error_Handler();

}

}

void MX_DMA_Init(void)

{

/* DMA controller clock enable */

__HAL_RCC_DMA2_CLK_ENABLE();

HAL_NVIC_SetPriority(DMA2_Stream0_IRQn, 7, 0);

HAL_NVIC_EnableIRQ(DMA2_Stream0_IRQn);

}

void MX_USART1_UART_Init(void)

{

huart1.Instance = USART1;

huart1.Init.BaudRate = 115200;

huart1.Init.WordLength = UART_WORDLENGTH_8B;

huart1.Init.StopBits = UART_STOPBITS_1;

huart1.Init.Parity = UART_PARITY_NONE;

huart1.Init.Mode = UART_MODE_TX_RX;

huart1.Init.HwFlowCtl = UART_HWCONTROL_NONE;

huart1.Init.OverSampling = UART_OVERSAMPLING_16;

if (HAL_UART_Init(&huart1) != HAL_OK)

{

Error_Handler();

}

}

#define BATCH_DATA_LEN 	3

/*main.c中的全局变量定义*/

uint32_t DataBuffer[BATCH_DATA_LEN];

/*main.h中的变量外扩及宏定义*/

extern uint32_t DataBuffer[BATCH_DATA_LEN];

/*DMA转换完成中断回调*/

void HAL_ADC_ConvCpltCallback(ADC_HandleTypeDef *hadc)

{

/*定时器DMA启动多通道转换*/

uint32_t val=0,Volt=0;

for(uint8_t i=0; i<BATCH_DATA_LEN; i++)

{

val=DataBuffer[i];

Volt=(3300*val)>>12;

}

}

int main(void)

{

HAL_Init();

SystemClock_Config();

MX_GPIO_Init();

MX_DMA_Init();

MX_ADC1_Init();

MX_USART1_UART_Init();

MX_FSMC_Init();

MX_TIM14_Init();

HAL_TIM_Base_Start_IT(&htim14);

HAL_TIM_PWM_Start(&htim14, TIM_CHANNEL_1);

drv_lcd_init();		// 初始化LCD

while (1)

{

lcd_clear(WHITE);

// 定义字符串用于显示

char adc_in16_str[32];

char adc_in17_str[32];

char adc_in18_str[32];

char temperature_str[32];

// 获取ADC电压值并转换为字符串

uint32_t adc_in16_volt = (3300 * DataBuffer[0]) >> 12;

uint32_t adc_in17_volt = (3300 * DataBuffer[1]) >> 12;

uint32_t adc_in18_volt = (3300 * DataBuffer[2]) >> 12;

// 电压转换为温度（单位：°C）

float temperature = (adc_in16_volt / 1000.0f - 0.76f) / 0.0025f; // 转换为温度 (单位：°C)

// 将电压值格式化为字符串

sprintf(adc_in16_str, "ADC_IN16: %.3f V", adc_in16_volt / 1000.0f);  // 毫伏 → 伏特

sprintf(adc_in17_str, "ADC_IN17: %.3f V", adc_in17_volt / 1000.0f);

sprintf(adc_in18_str, "ADC_IN18: %.3f V", adc_in18_volt / 1000.0f);

// 将温度格式化为字符串

sprintf(temperature_str, "Temp: %.2f C", temperature);

// 显示到LCD上

lcd_show_string(10, 20, 24, adc_in16_str);  // 显示 ADC_IN16 电压

lcd_show_string(10, 60, 24, adc_in17_str);  // 显示 ADC_IN17 电压

lcd_show_string(10, 100, 24, adc_in18_str); // 显示 ADC_IN18 电压

lcd_show_string(10, 140, 24, temperature_str); // 显示温度

HAL_ADC_Start_DMA(&hadc1, DataBuffer, BATCH_DATA_LEN);

HAL_Delay(1000);

/* USER CODE END WHILE */

/* USER CODE BEGIN 3 */

}

/* USER CODE END 3 */

}

六、当收到字符串“*LED ON*”时，点亮LED灯；当收到“*LED OFF*”时，熄灭LED灯；当收到“*BEEP ON*” 时，蜂鸣器发出救护车报警声；当收到“*BEEP OFF*”时，则关闭蜂鸣器。

要求：在LCD上实时显示指示灯和蜂鸣器的状态信息(ON/OFF)；

void MX_GPIO_Init(void)

{

GPIO_InitTypeDef GPIO_InitStruct = {0};

/* GPIO Ports Clock Enable */

__HAL_RCC_GPIOC_CLK_ENABLE();

__HAL_RCC_GPIOF_CLK_ENABLE();

__HAL_RCC_GPIOH_CLK_ENABLE();

__HAL_RCC_GPIOB_CLK_ENABLE();

__HAL_RCC_GPIOE_CLK_ENABLE();

__HAL_RCC_GPIOD_CLK_ENABLE();

__HAL_RCC_GPIOA_CLK_ENABLE();

__HAL_RCC_GPIOG_CLK_ENABLE();

/*Configure GPIO pin Output Level */

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_0, GPIO_PIN_RESET);

/*Configure GPIO pin Output Level */

HAL_GPIO_WritePin(GPIOF, LED_B_Pin|LED_R_Pin, GPIO_PIN_SET);

/*Configure GPIO pin Output Level */

HAL_GPIO_WritePin(GPIOD, GPIO_PIN_3, GPIO_PIN_SET);

/*Configure GPIO pins : KEY_LEFT_Pin KEY_DOWN_Pin KEY_RIGHT_Pin KEY_UP_Pin */

GPIO_InitStruct.Pin = KEY_LEFT_Pin|KEY_DOWN_Pin|KEY_RIGHT_Pin|KEY_UP_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_INPUT;

GPIO_InitStruct.Pull = GPIO_PULLUP;

HAL_GPIO_Init(GPIOC, &GPIO_InitStruct);

/*Configure GPIO pin : PB0 */

GPIO_InitStruct.Pin = GPIO_PIN_0;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_PULLDOWN;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

HAL_GPIO_Init(GPIOB, &GPIO_InitStruct);

/*Configure GPIO pins : LED_B_Pin LED_R_Pin */

GPIO_InitStruct.Pin = LED_B_Pin|LED_R_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_PULLUP;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_MEDIUM;

HAL_GPIO_Init(GPIOF, &GPIO_InitStruct);

/*Configure GPIO pin : PD3 */

GPIO_InitStruct.Pin = GPIO_PIN_3;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_PULLUP;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_MEDIUM;

HAL_GPIO_Init(GPIOD, &GPIO_InitStruct);

}

void MX_DMA_Init(void)

{

/* DMA controller clock enable */

__HAL_RCC_DMA2_CLK_ENABLE();

/* DMA interrupt init */

/* DMA2_Stream2_IRQn interrupt configuration */

HAL_NVIC_SetPriority(DMA2_Stream2_IRQn, 0, 0);

HAL_NVIC_EnableIRQ(DMA2_Stream2_IRQn);

}

void MX_USART1_UART_Init(void)

{

/* USER CODE BEGIN USART1_Init 0 */

/* USER CODE END USART1_Init 0 */

/* USER CODE BEGIN USART1_Init 1 */

/* USER CODE END USART1_Init 1 */

huart1.Instance = USART1;

huart1.Init.BaudRate = 115200;

huart1.Init.WordLength = UART_WORDLENGTH_8B;

huart1.Init.StopBits = UART_STOPBITS_1;

huart1.Init.Parity = UART_PARITY_NONE;

huart1.Init.Mode = UART_MODE_TX_RX;

huart1.Init.HwFlowCtl = UART_HWCONTROL_NONE;

huart1.Init.OverSampling = UART_OVERSAMPLING_16;

if (HAL_UART_Init(&huart1) != HAL_OK)

{

Error_Handler();

}

}

int fputc(int ch, FILE *f)

{

HAL_UART_Transmit(&huart1, (uint8_t *)&ch, 1, 1000);

return ch;

}

#define RX_BUFFER_SIZE 64

uint8_t rx_buffer[RX_BUFFER_SIZE];

uint8_t rx_buffer_flg = 0;

void UsartReceive_IDLE(UART_HandleTypeDef *huart)

{

uint32_t temp;

if((__HAL_UART_GET_FLAG(huart,UART_FLAG_IDLE) != RESET))  //如果数据收完了

{

__HAL_UART_CLEAR_IDLEFLAG(&huart1);

HAL_UART_DMAStop(&huart1);

rx_buffer_flg = 1;//接收完成标记

HAL_UART_Receive_DMA(&huart1, rx_buffer, RX_BUFFER_SIZE);

}

}

void HAL_UART_RxCpltCallback(UART_HandleTypeDef *huart) {

uint32_t tmp_flag;

if (huart->Instance == USART1) {

}

}

void delay(uint32_t i)

{

while(i--);

}

void sound1(void)

{

uint32_t i = 30000;

uint32_t start_time = HAL_GetTick();

while(HAL_GetTick() - start_time < 5000)

{

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_0, GPIO_PIN_SET);

delay(i);

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_0, GPIO_PIN_RESET);

delay(i);

i = i - 6;

if(i == 0)

i = 30000;

}

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_0, GPIO_PIN_RESET);

}

void sound2(void)

{

uint32_t i = 6000;

uint32_t start_time = HAL_GetTick();

while(HAL_GetTick() - start_time < 5000)

{

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_0, GPIO_PIN_RESET);

delay(i);

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_0, GPIO_PIN_SET);

delay(i);

i = i - 6;

if(i == 0)

i = 6000;

}

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_0, GPIO_PIN_RESET);

}

char display_str[50];//显示字符串

char LED_State = 0;//LED状态

char BEEP_State = 0;//蜂鸣器状态

int main(void)

{

HAL_Init();

SystemClock_Config();

MX_GPIO_Init();

MX_DMA_Init();

MX_FSMC_Init();

MX_TIM14_Init();

MX_USART1_UART_Init();

/* USER CODE BEGIN 2 */

HAL_TIM_Base_Start_IT(&htim14);

HAL_TIM_PWM_Start(&htim14, TIM_CHANNEL_1);

drv_lcd_init();

lcd_set_color(WHITE, BLACK);

__HAL_UART_ENABLE_IT(&huart1, UART_IT_IDLE); //使能IDLE中断

HAL_UART_Receive_DMA(&huart1, rx_buffer, RX_BUFFER_SIZE);

while (1)

{

if(rx_buffer_flg)

{//串口接收完毕

rx_buffer_flg = 0;

if(strstr(rx_buffer, "LED ON"))

{

LED_State = 1;

}

if(strstr(rx_buffer, "LED OFF"))

{

LED_State = 0;

}

if(strstr(rx_buffer, "BEEP ON"))

{

BEEP_State = 1;

}

if(strstr(rx_buffer, "BEEP OFF"))

{

BEEP_State = 0;

}

}

if(LED_State)

{//灯打开

lcd_show_string(50, 50, 32, "LED ON  ");

HAL_GPIO_WritePin(GPIOF, LED_B_Pin|LED_R_Pin, GPIO_PIN_RESET);

}

else

{//灯关闭

lcd_show_string(50, 50, 32, "LED OFF  ");

HAL_GPIO_WritePin(GPIOF, LED_B_Pin|LED_R_Pin, GPIO_PIN_SET);

}

if(BEEP_State)

{//蜂鸣器打开

lcd_show_string(50, 100, 32, "BEEP ON  ");

sound1();

}

else

{//蜂鸣器关闭

lcd_show_string(50, 100, 32, "BEEP OFF  ");

HAL_GPIO_WritePin(GPIOB, GPIO_PIN_0, GPIO_PIN_RESET);

}

HAL_Delay(100);

}

}

七、设置TIM2_CH4(PA3)产生PWM信号，然后通过TIM9_CH1(PA2)来测量该信号频率和占空比，同时将频率和占空比显示在LCD上，保留一位小数。

void MX_GPIO_Init(void)

{

GPIO_InitTypeDef GPIO_InitStruct = {0};

/* GPIO Ports Clock Enable */

__HAL_RCC_GPIOC_CLK_ENABLE();

__HAL_RCC_GPIOF_CLK_ENABLE();

__HAL_RCC_GPIOH_CLK_ENABLE();

__HAL_RCC_GPIOA_CLK_ENABLE();

__HAL_RCC_GPIOB_CLK_ENABLE();

__HAL_RCC_GPIOE_CLK_ENABLE();

__HAL_RCC_GPIOD_CLK_ENABLE();

__HAL_RCC_GPIOG_CLK_ENABLE();

/*Configure GPIO pin Output Level */

HAL_GPIO_WritePin(GPIOD, GPIO_PIN_3, GPIO_PIN_SET);

/*Configure GPIO pin : PD3 */

GPIO_InitStruct.Pin = GPIO_PIN_3;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_PULLUP;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_MEDIUM;

HAL_GPIO_Init(GPIOD, &GPIO_InitStruct);

}

void MX_FSMC_Init(void)

{

FSMC_NORSRAM_TimingTypeDef Timing = {0};

FSMC_NORSRAM_TimingTypeDef ExtTiming = {0};

hsram3.Instance = FSMC_NORSRAM_DEVICE;

hsram3.Extended = FSMC_NORSRAM_EXTENDED_DEVICE;

/* hsram3.Init */

hsram3.Init.NSBank = FSMC_NORSRAM_BANK3;

hsram3.Init.DataAddressMux = FSMC_DATA_ADDRESS_MUX_DISABLE;

hsram3.Init.MemoryType = FSMC_MEMORY_TYPE_SRAM;

hsram3.Init.MemoryDataWidth = FSMC_NORSRAM_MEM_BUS_WIDTH_8;

hsram3.Init.BurstAccessMode = FSMC_BURST_ACCESS_MODE_DISABLE;

hsram3.Init.WaitSignalPolarity = FSMC_WAIT_SIGNAL_POLARITY_LOW;

hsram3.Init.WrapMode = FSMC_WRAP_MODE_DISABLE;

hsram3.Init.WaitSignalActive = FSMC_WAIT_TIMING_BEFORE_WS;

hsram3.Init.WriteOperation = FSMC_WRITE_OPERATION_ENABLE;

hsram3.Init.WaitSignal = FSMC_WAIT_SIGNAL_DISABLE;

hsram3.Init.ExtendedMode = FSMC_EXTENDED_MODE_ENABLE;

hsram3.Init.AsynchronousWait = FSMC_ASYNCHRONOUS_WAIT_DISABLE;

hsram3.Init.WriteBurst = FSMC_WRITE_BURST_DISABLE;

hsram3.Init.PageSize = FSMC_PAGE_SIZE_NONE;

/* Timing */

Timing.AddressSetupTime = 15;

Timing.AddressHoldTime = 15;

Timing.DataSetupTime = 60;

Timing.BusTurnAroundDuration = 0;

Timing.CLKDivision = 16;

Timing.DataLatency = 17;

Timing.AccessMode = FSMC_ACCESS_MODE_A;

/* ExtTiming */

ExtTiming.AddressSetupTime = 9;

ExtTiming.AddressHoldTime = 15;

ExtTiming.DataSetupTime = 8;

ExtTiming.BusTurnAroundDuration = 0;

ExtTiming.CLKDivision = 16;

ExtTiming.DataLatency = 17;

ExtTiming.AccessMode = FSMC_ACCESS_MODE_A;

if (HAL_SRAM_Init(&hsram3, &Timing, &ExtTiming) != HAL_OK)

{

Error_Handler( );

}

/* USER CODE BEGIN FSMC_Init 2 */

/* USER CODE END FSMC_Init 2 */

}

void MX_TIM2_Init(void)

{

TIM_ClockConfigTypeDef sClockSourceConfig = {0};

TIM_MasterConfigTypeDef sMasterConfig = {0};

TIM_OC_InitTypeDef sConfigOC = {0};

htim2.Instance = TIM2;

htim2.Init.Prescaler = 2100-1;

htim2.Init.CounterMode = TIM_COUNTERMODE_UP;

htim2.Init.Period = 8000-1;

htim2.Init.ClockDivision = TIM_CLOCKDIVISION_DIV1;

htim2.Init.AutoReloadPreload = TIM_AUTORELOAD_PRELOAD_DISABLE;

sClockSourceConfig.ClockSource = TIM_CLOCKSOURCE_INTERNAL;

sMasterConfig.MasterOutputTrigger = TIM_TRGO_RESET;

sMasterConfig.MasterSlaveMode = TIM_MASTERSLAVEMODE_DISABLE;

sConfigOC.OCMode = TIM_OCMODE_PWM1;

sConfigOC.Pulse = 2000;

sConfigOC.OCPolarity = TIM_OCPOLARITY_HIGH;

sConfigOC.OCFastMode = TIM_OCFAST_DISABLE;

HAL_TIM_MspPostInit(&htim2);

}

void MX_TIM3_Init(void)

{

TIM_MasterConfigTypeDef sMasterConfig = {0};

TIM_OC_InitTypeDef sConfigOC = {0};

htim3.Instance = TIM3;

htim3.Init.Prescaler = 21-1;

htim3.Init.CounterMode = TIM_COUNTERMODE_UP;

htim3.Init.Period = 800-1;

htim3.Init.ClockDivision = TIM_CLOCKDIVISION_DIV1;

htim3.Init.AutoReloadPreload = TIM_AUTORELOAD_PRELOAD_DISABLE;

sMasterConfig.MasterOutputTrigger = TIM_TRGO_RESET;

sMasterConfig.MasterSlaveMode = TIM_MASTERSLAVEMODE_DISABLE;

sConfigOC.OCMode = TIM_OCMODE_PWM1;

sConfigOC.Pulse = 200;

sConfigOC.OCPolarity = TIM_OCPOLARITY_HIGH;

sConfigOC.OCFastMode = TIM_OCFAST_DISABLE;

__HAL_TIM_DISABLE_OCxPRELOAD(&htim3, TIM_CHANNEL_3);

HAL_TIM_MspPostInit(&htim3);

}

void MX_TIM9_Init(void)

{

TIM_SlaveConfigTypeDef sSlaveConfig = {0};

TIM_IC_InitTypeDef sConfigIC = {0};

htim9.Instance = TIM9;

htim9.Init.Prescaler = 8400-1;

htim9.Init.CounterMode = TIM_COUNTERMODE_UP;

htim9.Init.Period = 50000-1;

htim9.Init.ClockDivision = TIM_CLOCKDIVISION_DIV1;

htim9.Init.AutoReloadPreload = TIM_AUTORELOAD_PRELOAD_DISABLE;

sSlaveConfig.SlaveMode = TIM_SLAVEMODE_RESET;

sSlaveConfig.InputTrigger = TIM_TS_TI1FP1;

sSlaveConfig.TriggerPolarity = TIM_INPUTCHANNELPOLARITY_RISING;

sSlaveConfig.TriggerPrescaler = TIM_ICPSC_DIV1;

sSlaveConfig.TriggerFilter = 0;

sConfigIC.ICPolarity = TIM_INPUTCHANNELPOLARITY_RISING;

sConfigIC.ICSelection = TIM_ICSELECTION_DIRECTTI;

sConfigIC.ICPrescaler = TIM_ICPSC_DIV1;

sConfigIC.ICFilter = 0;

sConfigIC.ICPolarity = TIM_INPUTCHANNELPOLARITY_FALLING;

sConfigIC.ICSelection = TIM_ICSELECTION_INDIRECTTI;

}

void MX_TIM14_Init(void)

{

TIM_OC_InitTypeDef sConfigOC = {0};

htim14.Instance = TIM14;

htim14.Init.Prescaler = 21-1;

htim14.Init.CounterMode = TIM_COUNTERMODE_UP;

htim14.Init.Period = 4000-1;

htim14.Init.ClockDivision = TIM_CLOCKDIVISION_DIV1;

htim14.Init.AutoReloadPreload = TIM_AUTORELOAD_PRELOAD_DISABLE;

sConfigOC.OCMode = TIM_OCMODE_PWM1;

sConfigOC.Pulse = 3200;

sConfigOC.OCPolarity = TIM_OCPOLARITY_HIGH;

sConfigOC.OCFastMode = TIM_OCFAST_DISABLE;

HAL_TIM_MspPostInit(&htim14);

}

__IO uint32_t	IC2ValuePWM = 0;

__IO uint32_t	DutyCyclePWM = 0;

__IO uint32_t	FrequencyPWM = 0;

__IO uint32_t	FrequencyPWMx = 0;

// TIM2产生的PWM周期为0.2s，占空比为25%，TIM9的时钟频率为20KHz, CounterPeriod为50000-1

void HAL_TIM_IC_CaptureCallback(TIM_HandleTypeDef *htim)

{

if(htim->Instance == TIM9 && htim->Channel == HAL_TIM_ACTIVE_CHANNEL_1)

{

printf("TIM9 Capture Interrupt!\n"); // 通过串口打印

// 获取输入波形的捕获值

IC2ValuePWM = HAL_TIM_ReadCapturedValue(htim, TIM_CHANNEL_1);

if(IC2ValuePWM != 0)

{

// 计算PWM信号占空比

DutyCyclePWM = ((HAL_TIM_ReadCapturedValue(htim, TIM_CHANNEL_2)) * 100)/ IC2ValuePWM;// (高电平时间 / 总周期) × 100%

// 计算PWM信号频率

FrequencyPWM = (HAL_RCC_GetPCLK2Freq()*2/(TIM9->PSC+1))/IC2ValuePWM;

}

else

{

DutyCyclePWM = 0;

FrequencyPWM= 0;

}

}

}

void show_pwm_info(float duty_cycle, float frequency)

{

char buffer[32];

// 显示占空比

sprintf(buffer, "Duty: %.1f % %", duty_cycle);

lcd_show_string(0, 0, 32, buffer);

// 显示频率

sprintf(buffer, "Freq: %.1f Hz", frequency);

lcd_show_string(0, 32, 32, buffer);

}

int main(void)

{

uint8_t		dir = 1;

uint16_t 	Duty = 0;

HAL_Init();

SystemClock_Config();

MX_GPIO_Init();

MX_FSMC_Init();

MX_TIM14_Init();

MX_USART1_UART_Init();

MX_TIM2_Init();

MX_TIM9_Init();

HAL_TIM_Base_Start_IT(&htim2);

HAL_TIM_PWM_Start(&htim2, TIM_CHANNEL_4);

HAL_TIM_Base_Start_IT(&htim14);

HAL_TIM_PWM_Start(&htim14, TIM_CHANNEL_1);

HAL_NVIC_SetPriority(TIM1_BRK_TIM9_IRQn, 5, 0);

HAL_NVIC_EnableIRQ(TIM1_BRK_TIM9_IRQn);

HAL_TIM_IC_Start_IT(&htim9, TIM_CHANNEL_1);

HAL_TIM_IC_Start_IT(&htim9, TIM_CHANNEL_2);

drv_lcd_init();

while (1)

{

HAL_Delay(1000);

show_pwm_info(DutyCyclePWM,FrequencyPWM );

}

}

八、选择TIM6基本定时器实现精确的1秒的定时，将时、分、秒信息显示在LCD上，通过按键可实现时（左键）、分（下键）、秒（右键）调整。时间初值为08:10:00。

要求： 按键通过TIM7进行定时扫描，同时带有消抖功能；

#include <rtthread.h>

#include <rtdevice.h>

#include <board.h>

#include <drv_lcd.h>

#include <stdio.h>

/* 硬件按键定义 */

#define BTN_LEFT_PIN     GET_PIN(C, 0)  // PC0

#define BTN_DOWN_PIN     GET_PIN(C, 1)  // PC1

#define BTN_RIGHT_PIN    GET_PIN(C, 4)  // PC4

/* 时钟结构体 */

typedef struct {

volatile uint8_t hours;

volatile uint8_t minutes;

volatile uint8_t seconds;

} ClockTime;

/* 按键状态结构体 */

typedef struct {

uint8_t last_state;

uint8_t current_state;

uint8_t pin;

} Button;

/* 全局变量 */

static ClockTime current_time = {8, 10, 0};

static Button buttons[] = {

{1, 1, BTN_LEFT_PIN},   // 左键

{1, 1, BTN_DOWN_PIN},   // 下键

{1, 1, BTN_RIGHT_PIN}   // 右键

};

/* LCD显示函数 */

static void lcd_show_time(void)

{

char time_str[10];

rt_snprintf(time_str, sizeof(time_str), "%02d:%02d:%02d",

current_time.hours, current_time.minutes, current_time.seconds);

lcd_show_string(56, 104, 32, time_str);

}

/* 时钟更新函数 */

static void update_clock(void)

{

current_time.seconds++;

if (current_time.seconds >= 60)

{

current_time.seconds = 0;

current_time.minutes++;

if (current_time.minutes >= 60)

{

current_time.minutes = 0;

current_time.hours = (current_time.hours + 1) % 24;

}

}

}

/* TIM6中断 - 时钟更新 */

void TIM6_DAC_IRQHandler(void)

{

if (TIM6->SR & TIM_SR_UIF)

{

TIM6->SR &= ~TIM_SR_UIF;

update_clock();

}

}

/* 按键处理函数 */

static void handle_button_press(Button *btn, uint8_t *time_value, uint8_t max_value)

{

if (btn->last_state == 1 && btn->current_state == 0) // 检测下降沿

{

rt_thread_mdelay(20); // 20ms消抖延时

if (rt_pin_read(btn->pin) == 0) // 确认仍然按下

{

*time_value = (*time_value + 1) % max_value;

while (rt_pin_read(btn->pin) == 0); // 等待按键释放

rt_thread_mdelay(20); // 释放消抖

}

}

btn->last_state = btn->current_state;

}

/* 按键扫描处理 */

static void scan_buttons(void)

{

/* 左键 - 调整小时 */

buttons[0].current_state = rt_pin_read(buttons[0].pin);

handle_button_press(&buttons[0], &current_time.hours, 24);

/* 下键 - 调整分钟 */

buttons[1].current_state = rt_pin_read(buttons[1].pin);

handle_button_press(&buttons[1], &current_time.minutes, 60);

/* 右键 - 调整秒钟 */

buttons[2].current_state = rt_pin_read(buttons[2].pin);

handle_button_press(&buttons[2], &current_time.seconds, 60);

}

/* TIM7中断 - 按键扫描 */

void TIM7_IRQHandler(void)

{

if (TIM7->SR & TIM_SR_UIF)

{

TIM7->SR &= ~TIM_SR_UIF;

scan_buttons();

}

}

/* 定时器初始化 */

static void timer_config(void)

{

/* TIM6 时钟配置 (1秒中断) */

__HAL_RCC_TIM6_CLK_ENABLE();

TIM6->PSC = 8399;

TIM6->ARR = 9999;

TIM6->DIER |= TIM_DIER_UIE;

TIM6->CR1 |= TIM_CR1_CEN;

TIM6->EGR = TIM_EGR_UG;  // 手动触发更新，确保配置生效

TIM6->SR = 0;            // 清除所有状态标志

NVIC_SetPriority(TIM6_DAC_IRQn, 2);

NVIC_EnableIRQ(TIM6_DAC_IRQn);

/* TIM7 按键扫描配置 (10ms中断) */

__HAL_RCC_TIM7_CLK_ENABLE();

TIM7->PSC = 839;

TIM7->ARR = 99;

TIM7->DIER |= TIM_DIER_UIE;

TIM7->CR1 |= TIM_CR1_CEN;

NVIC_SetPriority(TIM7_IRQn, 3);

NVIC_EnableIRQ(TIM7_IRQn);

}

/* 硬件初始化 */

static void hardware_init(void)

{

rt_pin_mode(BTN_LEFT_PIN, PIN_MODE_INPUT_PULLUP);

rt_pin_mode(BTN_DOWN_PIN, PIN_MODE_INPUT_PULLUP);

rt_pin_mode(BTN_RIGHT_PIN, PIN_MODE_INPUT_PULLUP);

timer_config();

}

/* 主函数 */

int main(void)

{

/* 初始化 */

hardware_init();

lcd_show_time();

/* 主循环 */

uint8_t last_second = current_time.seconds;

while (1)

{

if (current_time.seconds != last_second)

{

lcd_show_time();

last_second = current_time.seconds;

}

rt_thread_mdelay(10); // 10ms循环周期

}

return 0;

}

九、设计程序将自己的中文姓名、学号和证件照显示在LCD上。

void MX_USART1_UART_Init(void)

{

huart1.Instance = USART1;

huart1.Init.BaudRate = 115200;

huart1.Init.WordLength = UART_WORDLENGTH_8B;

huart1.Init.StopBits = UART_STOPBITS_1;

huart1.Init.Parity = UART_PARITY_NONE;

huart1.Init.Mode = UART_MODE_TX_RX;

huart1.Init.HwFlowCtl = UART_HWCONTROL_NONE;

huart1.Init.OverSampling = UART_OVERSAMPLING_16;

if (HAL_UART_Init(&huart1) != HAL_OK)

{

Error_Handler();

}

}

十、基于RT-Thread操作系统编写程序，实现如下功能：

1 任务1，将读取AHT21的温湿度信息，小数点后保留1位，显示在LCD屏上，300ms刷新一次；

2 任务2，将串口接收的字符串显示在LCD上，字符串长度不超过20个字节，采用DMA结合信号量方式接收；

3 轻触一次DOWN按键，中断回调函数里翻转红色LED灯；

#include <rtthread.h>

#include <rtdevice.h>

#include <board.h>

#include <aht10.h>

#include <drv_lcd.h>

#include <string.h>

#include <stdio.h>

#define DBG_TAG "main"

#define DBG_LVL DBG_LOG

#include <rtdbg.h>

/* 硬件定义 */

#define LED_PIN         GET_PIN(F, 12)    // 红灯引脚定义 (根据你的硬件平台修改)

#define BUTTON_PIN      GET_PIN(C, 1)     // DOWN按键引脚定义 (根据你的硬件平台修改)

#define UART_NAME       "uart1"         // 使用的串口设备名称 (根据你的硬件平台修改)

#define USART_REC_LEN   64              // 串口接收缓冲区长度

#define AHT10_I2C_BUS   "i2c3"          // AHT10传感器连接的I2C总线名称 (根据你的硬件平台修改)

/* 全局变量 */

static aht10_device_t aht10_dev = RT_NULL; // AHT10设备句柄，用于访问AHT10传感器

static rt_device_t uart_dev = RT_NULL;     // 串口设备句柄，用于串口通信

static struct rt_semaphore rx_sem;       // 接收信号量，用于通知串口接收线程有数据到达

static rt_mutex_t display_mutex = RT_NULL; // LCD显示互斥锁，用于保护对LCD的并发访问

static char uart_rx_buf[USART_REC_LEN + 1] = {0}; // 串口接收缓冲区，用于存储接收到的数据

/* 串口中断回调函数 */

static rt_err_t uart_rx_callback(rt_device_t dev, rt_size_t size)

{

static rt_size_t rx_index = 0; // 静态变量，用于记录当前接收缓冲区的位置

if (size > 0) // 如果接收到数据

{

char ch;

while (rt_device_read(uart_dev, 0, &ch, 1) == 1)  // 逐字节读取接收到的数据

{

if (rx_index >= USART_REC_LEN - 1)  // 如果接收缓冲区即将满了（预留一个位置给字符串结束符）

{

rx_index = 0;  // 复位索引，覆盖旧的数据，防止缓冲区溢出

}

uart_rx_buf[rx_index++] = ch; // 将接收到的字符存储到接收缓冲区

if (ch == ' ')  // 检测到空格字符，认为一个完整的字符串接收完毕 (你可以根据实际需求修改结束符)

{

uart_rx_buf[rx_index - 1] = '\0'; // 在接收到的字符串末尾添加字符串结束符

rt_sem_release(&rx_sem); // 释放接收信号量，通知串口接收线程处理数据

rx_index = 0; // 复位索引，准备接收下一个字符串

break; // 停止当前读取，等待信号量被获取

}

}

}

return RT_EOK;

}

/* 串口接收线程 */

static void uart_rx_thread_entry(void *parameter)

{

while (1) // 无限循环

{

/* 等待接收信号量，如果接收到信号量，表示串口接收到了以空格结尾的字符串 */

if (rt_sem_take(&rx_sem, RT_WAITING_FOREVER) == RT_EOK)

{

/* 获取LCD显示互斥锁，确保在操作LCD时不会被其他任务打断 */

rt_mutex_take(display_mutex, RT_WAITING_FOREVER);

// 显示接收到的数据到控制台 (用于调试)

rt_kprintf("Received: %s\n", uart_rx_buf);

lcd_clear(WHITE); // 清空LCD屏幕，防止之前显示的内容残留

//lcd_show_string(10, 20, 24, uart_rx_buf); // 在LCD屏幕的指定位置显示接收到的字符串

//lcd_fill(10, 0, 10 + 16 * 20, 0 + 16, WHITE);  // 清除显示区域

lcd_show_string(10, 30, 32, (char*)uart_rx_buf); // 显示新字符串

/* 释放LCD显示互斥锁 */

rt_mutex_release(display_mutex);

}

}

}

/* 按键中断回调 */

static void button_irq_callback(void *args)

{

/* 翻转红色LED灯的状态 (读取当前状态，然后取反写入) */

rt_pin_write(LED_PIN, !rt_pin_read(LED_PIN));

}

/* 温湿度读取线程 */

static void sensor_thread_entry(void *param)

{

float temperature, humidity;

char temp_str[20], humi_str[20];

while (1) { // 无限循环

/* 读取AHT21的温度值 */

temperature = aht10_read_temperature(aht10_dev);

/* 读取AHT21的湿度值 */

humidity = aht10_read_humidity(aht10_dev);

/* 获取LCD显示互斥锁 */

rt_mutex_take(display_mutex, RT_WAITING_FOREVER);

/* 格式化温度字符串，保留一位小数 */

snprintf(temp_str, sizeof(temp_str), "Temp: %.1f C", temperature);

/* 格式化湿度字符串，保留一位小数 */

snprintf(humi_str, sizeof(humi_str), "Humi: %.1f %%", humidity);

/* 在LCD屏幕上显示温度信息 */

lcd_show_string(10, 80, 32, temp_str);

/* 在LCD屏幕上显示湿度信息 */

lcd_show_string(10, 120, 32, humi_str);

/* 释放LCD显示互斥锁 */

rt_mutex_release(display_mutex);

/* 延时300毫秒 */

rt_thread_mdelay(300);

}

}

int main(void)

{

/* 初始化LED引脚为输出模式 */

rt_pin_mode(LED_PIN, PIN_MODE_OUTPUT);

/* 初始化DOWN按键引脚为上拉输入模式 */

rt_pin_mode(BUTTON_PIN, PIN_MODE_INPUT_PULLUP);

/* 绑定按键中断回调函数，当按键按下（下降沿）时执行 button_irq_callback */

rt_pin_attach_irq(BUTTON_PIN, PIN_IRQ_MODE_FALLING, button_irq_callback, RT_NULL);

/* 使能按键中断 */

rt_pin_irq_enable(BUTTON_PIN, PIN_IRQ_ENABLE);

lcd_clear(WHITE); // 清空LCD屏幕，设置背景为白色

/* 初始化AHT10传感器，返回设备句柄 */

aht10_dev = aht10_init(AHT10_I2C_BUS);

/* 初始化串口设备 */

uart_dev = rt_device_find(UART_NAME);

if (uart_dev) {

/* 以DMA接收模式打开串口设备 */

rt_device_open(uart_dev, RT_DEVICE_FLAG_DMA_RX);

/* 设置串口接收中断回调函数，当串口接收到数据时，执行 uart_rx_callback */

rt_device_set_rx_indicate(uart_dev, uart_rx_callback);

/* 初始化接收信号量，初始值为0 */

rt_sem_init(&rx_sem, "rx_sem", 0, RT_IPC_FLAG_FIFO);

}

/* 创建互斥锁，用于保护LCD的访问 */

display_mutex = rt_mutex_create("disp_mtx", RT_IPC_FLAG_FIFO);

/* 启动线程 */

if (sensor_thread != RT_NULL)

rt_thread_startup(sensor_thread);

if (uart_thread != RT_NULL)

rt_thread_startup(uart_thread);

while (1) {

/* 主线程可以执行一些低优先级的任务，或者进入休眠 */

rt_thread_mdelay(10);

}

return RT_EOK;

}