实验三   存储器实验

一、实验目的

1、了解随机存取存储器 RAM 的原理。

2、理解 RAM 读取、写入数据的过程。

3、理解计算机中存储器地址编址和数据索引方法。

4、熟悉并运用 verilog 语言进行电路设计。

5、 为后续设计 cpu 的实验打下基础。

二、实验设备

1、装有 Xilinx Vivado的计算机一台

2、 LS-CPU-EXB-002教学系统实验箱一套

三、实验内容

1、学习存储器的设计及原理和计算机中内存地址编址和数据索引方法。

2 、搭建一个异步的RAM，能够正确地将数据写入指定地址并读出。

3、将以上设计作为一个单独的模块，设计一个外围模块去调用它，外围模块中需要调用封装好的触摸屏模块。

4、向地址为50H-55H的六个单元送70H-75H六个数，验收时任意指定一个地址，屏幕上输出其存储的数据。

5、进行综合布局布线，下载到实验箱的FPGA板上进行演示。

四、实验步骤

1、启动Vivado，选择File->New Project，输入工程名称，选择工程文件的路径

2、选择RTL Project，勾选Do not specify sources at this time

3、在筛选器中进行如下选择：

family->Artix7 package->fbg676  最后选择型号xc7a200tfbg676-2

4、添加源文件Project Manager->Add sources->Add or create design sources

->Create File,开始输入程序代码

5、添加外围模块，该模块调用加法器和触摸屏模块以便在实验箱上输入原始数据和输

出结果。

6、将封装好的触摸屏模块添加进工程，用户只需要直接调用。继续选择Add sources，

添加lcd_module.dcp.

7、添加引脚绑定的约束文件，扩展名为.xdc。

8、也可以通过Vivado中的I/O Planning功能来产生约束文件。

点击Flow Navigator中Synthesis下的Run Synthesis，进行综合，综合完成后选择Open Synthesized Design，查看综合结果。参照实验板原理图和引脚对应关系表，在I/O Ports栏下输入正确的Package Pin，针对本实验板，I/O Std要统一设置为LVCM0S33。设置完成保存为约束文件。

9、综合、布局布线并产生可烧写文件，扩展名为.bit。可以依次双击运行，也可以只双击Generate Bitstream，会自动运行这三步。

10、打开实验箱电源，在比特流文件生成完成的窗口选择Open Hardware Manager，进入硬件管理界面。在Hardware Manager窗口的提示信息中，点击Open Target，选择Auto Connect自动连接器件。

11、对目标硬件编程，在Hardware窗口右键单击目标器件cx7a200t_0，选择Program Device…，选择下载的bit流文件，点击Program，完成下载后，窗口下的cx7a200t_0状态变成Programmed。

12、通过触摸屏完成数据输入和结果显示，验证实验的正确性。

五、记录实验结果。

六、撰写实验报告

dream_v:

`timescale 1ns / 1ps

module dcram(

input clk,

input we,

input [7:0] addr,

input [31:0] datain,

output [31:0] dataout

);

reg [31:0]  ram[0:127];

assign dataout  = ram[addr];

always @(posedge clk) begin

if(we)  ram[addr] <= datain;

end

integer i;

initial begin

for (i = 0; i < 128; i = i+1)

ram[i] = 0;

ram[8'h50] = 32'h70;

ram[8'h51] = 32'h71;

ram[8'h52] = 32'h72;

ram[8'h53] = 32'h73;

ram[8'h54] = 32'h74;

ram[8'h55] = 32'h75;

end

endmodule

display:

`timescale 1ns / 1ps

module dcram_display(

//时钟与复位信号

input clk,

input resetn,       //后缀"n"代表低电平有效

//拨码开关，用于选择输入数和产生cin

input we_sel,       //0:读;1:写

input input_sel,    //0:addr;1:datain

//led灯，用于显示cout

output led_cout,

//触摸屏相关接口，不需要更改

output lcd_rst,

output lcd_cs,

output lcd_rs,

output lcd_wr,

output lcd_rd,

inout[15:0] lcd_data_io,

output lcd_bl_ctr,

inout ct_int,

inout ct_sda,

output ct_scl,

output ct_rstn

);

//-----{调用模块}begin

wire  we;

reg [7:0] addr;

reg [31:0] datain;

wire [31:0] dataout;

dcram dcram_module (.clk(clk), .we(we), .addr(addr), .datain(datain), .dataout(dataout));

assign we = we_sel;

//-----{调用模块}end\

//---------------------{调用触摸屏模块}begin--------------------//

//-----{实例化触摸屏}begin

//此小节不需要更改

reg         display_valid;

reg  [39:0] display_name;

reg  [31:0] display_value;

wire [5 :0] display_number;

wire        input_valid;

wire [31:0] input_value;

lcd_module lcd_module(

.clk            (clk           ),   //10Mhz

.resetn         (resetn        ),

//调用触摸屏的接口

.display_valid  (display_valid ),

.display_name   (display_name  ),

.display_value  (display_value ),

.display_number (display_number),

.input_valid    (input_valid   ),

.input_value    (input_value   ),

//lcd触摸屏相关接口，不需要更改

.lcd_rst        (lcd_rst       ),

.lcd_cs         (lcd_cs        ),

.lcd_rs         (lcd_rs        ),

.lcd_wr         (lcd_wr        ),

.lcd_rd         (lcd_rd        ),

.lcd_data_io    (lcd_data_io   ),

.lcd_bl_ctr     (lcd_bl_ctr    ),

.ct_int         (ct_int        ),

.ct_sda         (ct_sda        ),

.ct_scl         (ct_scl        ),

.ct_rstn        (ct_rstn       )

);

//-----{实例化触摸屏}end

//-----{从触摸屏获取输入}begin

//根据实际需要输入的数修改此小节，

//建议对每一个数的输入，编写单独一个always块

always @(posedge clk) begin

if (!resetn) begin

addr <= 8'd0;

end

else if (input_valid && !input_sel) begin

addr <= input_value;

end

end

always @(posedge clk) begin

if (!resetn) begin

datain <= 32'd0;

end

else if (input_valid && input_sel) begin

datain <= input_value;

end

end

//-----{从触摸屏获取输入}end

//-----{输出到触摸屏显示}begin

//根据需要显示的数修改此小节，

//触摸屏上共有44块显示区域，可显示44组32位数据

//44块显示区域从1开始编号，编号为1~44，

always @(posedge clk)

begin

case(display_number)

6'd1 :

begin

display_valid <= 1'b1;

display_name  <= "ADDR";

display_value <= addr;

end

6'd11 :

begin

display_valid <= 1'b1;

display_name  <= "IN";

display_value <= datain;

end

6'd2 :

begin

display_valid <= 1'b1;

display_name  <= "OUT";

display_value <= dataout;

end

default :

begin

display_valid <= 1'b0;

display_name  <= 40'd0;

display_value <= 32'd0;

end

endcase

end

//-----{输出到触摸屏显示}end

//----------------------{调用触摸屏模块}end---------------------//

Endmodule

register：

set_property IOSTANDARD LVCMOS33 [get_ports {lcd_data_io[13]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lcd_data_io[12]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lcd_data_io[11]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lcd_data_io[10]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lcd_data_io[9]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lcd_data_io[8]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lcd_data_io[7]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lcd_data_io[6]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lcd_data_io[5]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lcd_data_io[4]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lcd_data_io[3]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lcd_data_io[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lcd_data_io[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lcd_data_io[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lcd_data_io[15]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lcd_data_io[14]}]

set_property IOSTANDARD LVCMOS33 [get_ports ct_int]

set_property IOSTANDARD LVCMOS33 [get_ports ct_rstn]

set_property IOSTANDARD LVCMOS33 [get_ports ct_scl]

set_property IOSTANDARD LVCMOS33 [get_ports ct_sda]

set_property IOSTANDARD LVCMOS33 [get_ports input_sel]

set_property IOSTANDARD LVCMOS33 [get_ports lcd_bl_ctr]

set_property IOSTANDARD LVCMOS33 [get_ports lcd_cs]

set_property IOSTANDARD LVCMOS33 [get_ports lcd_rd]

set_property IOSTANDARD LVCMOS33 [get_ports lcd_rst]

set_property IOSTANDARD LVCMOS33 [get_ports lcd_rs]

set_property IOSTANDARD LVCMOS33 [get_ports lcd_wr]

set_property IOSTANDARD LVCMOS33 [get_ports led_cout]

set_property IOSTANDARD LVCMOS33 [get_ports resetn]

set_property IOSTANDARD LVCMOS33 [get_ports we_sel]

set_property IOSTANDARD LVCMOS33 [get_ports clk]

set_property PACKAGE_PIN AD24 [get_ports input_sel]

set_property PACKAGE_PIN J25 [get_ports lcd_rst]

set_property PACKAGE_PIN H18 [get_ports lcd_cs]

set_property PACKAGE_PIN K16 [get_ports lcd_rs]

set_property PACKAGE_PIN L8 [get_ports lcd_wr]

set_property PACKAGE_PIN K8 [get_ports lcd_rd]

set_property PACKAGE_PIN J15 [get_ports lcd_bl_ctr]

set_property PACKAGE_PIN L19 [get_ports ct_int]

set_property PACKAGE_PIN G24 [get_ports ct_rstn]

set_property PACKAGE_PIN H21 [get_ports ct_scl]

set_property PACKAGE_PIN J24 [get_ports ct_sda]

set_property PACKAGE_PIN H9 [get_ports {lcd_data_io[0]}]

set_property PACKAGE_PIN K17 [get_ports {lcd_data_io[1]}]

set_property PACKAGE_PIN J20 [get_ports {lcd_data_io[2]}]

set_property PACKAGE_PIN M17 [get_ports {lcd_data_io[3]}]

set_property PACKAGE_PIN L17 [get_ports {lcd_data_io[4]}]

set_property PACKAGE_PIN L18 [get_ports {lcd_data_io[5]}]

set_property PACKAGE_PIN L15 [get_ports {lcd_data_io[6]}]

set_property PACKAGE_PIN M15 [get_ports {lcd_data_io[7]}]

set_property PACKAGE_PIN M16 [get_ports {lcd_data_io[8]}]

set_property PACKAGE_PIN L14 [get_ports {lcd_data_io[9]}]

set_property PACKAGE_PIN M14 [get_ports {lcd_data_io[10]}]

set_property PACKAGE_PIN F22 [get_ports {lcd_data_io[11]}]

set_property PACKAGE_PIN G22 [get_ports {lcd_data_io[12]}]

set_property PACKAGE_PIN G21 [get_ports {lcd_data_io[13]}]

set_property PACKAGE_PIN H24 [get_ports {lcd_data_io[14]}]

set_property PACKAGE_PIN J16 [get_ports {lcd_data_io[15]}]

set_property PACKAGE_PIN Y3 [get_ports resetn]

set_property PACKAGE_PIN AC19 [get_ports clk]

set_property PACKAGE_PIN H7 [get_ports led_cout]

set_property PACKAGE_PIN AC21 [get_ports we_sel]